State Feedback System Control for Human Inspiratory System

Lafta E.Jumaa Alkurawy1, Khalid G. Mohammed , Ammar Issa Ismail3

, 1Department of Electronics Engineering, Department of Electrical Power and Machine Engineering

Universitas of Diyala, raq

| Article Info |  | ABSTRACT (10 PT) |
| --- | --- | --- |
| Article history: Received Sep 9, 2019 Revised May 20, 2020 Accepted Jun 11, 2020 |  | The Mathematical modeling of human respiratory system is an essentially part in saving precision information of diagnostic about the disease of cardiovascular respiratory system. The physics of respiratory system and cardiovascular are completely interconnected with each other. In this paper, we will study the state feedback control for the inspiratory system during study the characteristics of the response output with the stability. The model of system is nonlinear and linearized it by Tayler method to be simple to matching with the control theory. We convert the system from differential equation to state equation to find the optimal control that helps to drive the respiratory system. Simulations are managed to indicate the proposed method effectiveness. The results of simulations are validated by using a real information form the health center. |
| Keywords: modelin ,nonlinear respiratory system Control |  |  |
|  |  | Copyright © 2020 Institute of Advanced Engineering and Science. All rights reserved. |
| Corresponding Author: Lafta Esmaeel Jumaa Alkurawy, Department of Electronics Engineering, University of Diyala, Quds Square, Baquba,Diyala, Iraq. Email: lafta_67@yahoo.com |  |  |

- INTRODUCTION

The diseases in the World Health Organization of  non-communicable like diseases of respiratory are causing of deaths in the hospitals. The diseases of respiratory and cardiovascular  can be predicted with  real tools and diagnostic information. The mathematical model of  respiratory system  will assist in supplying more information of  diagnostic. It permit to make the complex interaction be accuracy between several systems, and estimate the certain diseases which change the function of normal system as shown in Fig 1.

Fig. 1 The human respiratory system

Ireneusz Jabłoński and Janusz Mroczka[1], they have presented a model reduction for respiratory system focused on electrical educed circuit. The chief technique depends on a numerical procedure for parametric and structural analysis that applied to equivalent of forward linear which designed for interrupter experiment conditions. The results was reliable in the frequency and time domain for response of dynamic behavior.

Behrang Amini et al. [2], they have developed a model of the central pattern for respiratory system to mimic the medullary neurons salient characteristics. The model has designed and dived by neuron of pacemaking  from complex of pre-Botzinge. The results of this model have been focused on data of voltage clamp.

Clara M. Ionescu et al. [3], they have provided the respiratory airway mechanical parameters  to make easy the correlation the pathologic changes with fractional-order models. This technique involves of tube length, wall thickness and inner radius for level of airway to collect them into equations for modeling the wall elasticity, flow air velocity, and pressure drop. The parameters of mechanical have derived offer the capability to evaluate the impedance of input by varying the parameters morphology to the disease of pulmonary.

Nicolas W. Chbat   et al. [4], they have developed a model of comprehensive cardiopulmonary to take into account the relationship between the respiratory and the cardiovascular systems. This model involves the gas exchange, lung mechanics,  heart, control of cardio-ventilatory and equations of transport. The results have showed good acceptable with date of published patients in case of the simulation of hyperoxic and normoxic.

Clara Mihaela Ionescu  et al. [5], they have developed a model to demonstrate the relation between the fractional – order model appearance and the respiratory tree recurrence. This model is enable of simulating the lungs mechanical properties and did compare this model with measurement of vivo for the respiratory input impedance in 20 healthy patients. They have provided an additional of the fractal for  consequence appearance and human lungs in impedance of the total respiratory system.

He zhonghai  [6], he has designed a model  for sponeous breath and pressure of respiratory muscle. The pressure of respiratory muscle for autonomous respiration  is utilized in the construction of the model. They have got a good results with the tool and flow of respiratory.

Mario A. Quiroz-Ju´arez et al. [7], they have presented a model with heterogeneous oscillator of cardiac system for ECG waveform.  The model has consisted the pacemarkes of main natural that has represented by equations of van der Pol and ventricular and atrial muscles. They have included an artificial RR-tachogram with the statistics of frequency domain and heart rate by waves of respiratory and Mayer. The standard 12 ECG has computed by the signals of ventricle and atria and this model has been fitted for real patients by clinical ECG.

Chi-Hsuan Hsieh et al. [8], they have presented  a radar signal of ultra-wideband impulse radio utilized to the features of human respiratory system. They have modified raised a waveform of cosine respiration model that could obtain extra features of inspiratory system such as speed of expiation and inspiration, ratio of respiration holding and respiration intensity. This model has suggested the signal of measured respiration that compared with the model of MRCW and reduced the complexity of computation. The results have showed that system can detect the respiration rates of human at 0.1 to 1 Hz in the real time analyzing.

Michael Revow et al. [9], they have provided a framework for automatic neonatal respiratory control has been designed to help the control of respiratory during the period of newborn. A model with perturbation analysis has determined the response of dynamic ventilator was the sensitive to affect  the loop of peripheral chemo reflex. The results have showed that the model provides valuable during the period of neonatal and provides a method  to examine the human infants in clinic units.

Bailiang Chen et al. [10], they have developed a model and corrected motion of respiratory for free breathing imaging of MR. The respiration was modeled and estimated from sensor of MARMOT and belts of Pneumatic, focused on images of real-time and method of regression. The accuracy of sensor was validate by comparing  errors of motion the kidney. The results was compatible for sensors with environment of MR and the errors of motion of modeling and estimation with sensors were compatible.

- MODELING THE RESPIRATORY SYSYEM

## The model of respiratory system in[11-13], the differential equations as

VA dpadt=863 Fi Cv-Ca+dpAdt(Pi-Pa)                     (1)

R= ∆PaQ                                                                             (2)

V dcvdt=MR+ Fi(Ca-Cv)                                             (3)

Ca dpadt=Q- Fi                                                               (4)

Cv dpadt=Fi-Q                                                                (5)

Gp= ∆PA∆PI ∆Pa∆PA  ∆PI ∆FI                                                             (6)

Gp=( 1-xd ) VI8.63 1-ysQ  βc+ 1- xd  VI  1-ysβcβa (PB- PH2o).     (7)

τ= VA 8.63 1-ysQP  βc+ 1- xd  VI                                           (8)

τ=VA VA+ 8.63 QP  βc.                                                               (9)

The changing of oxygen saturation SpO2 to inspiration fraction Fio2 is

τ ∆SpO2 s+ ∆SpO2= Gpc ∆Fio2.                                 (10)

The transfer function of output SpO2 to  Fio2 is

VAVA+8.63 QP ∆SpO2s+ ∆SpO2= 1-xdVI 1-ysβc8.631-ysQβa+1-xdVI ∆FiO2                                      (11)

The response of oxygen saturation of  SpO2for the a mature human will be started form 90-95 when applied Fio2 from 20% - 40% as shown in Fig. 2.

Fig. 2. Output SpO2 for human respiratory when FiO2 input.

- State Feedback Control System

The dynamical system state is an accumulate of variables that allows estimation the response of future system. The designing the system to be controlled is depicted by a linear state model for the human respiratory and has single input. The control of feedback will be developed by the positioning the eigenvalues of closed loop in required locations.

The typical control system by using state feedback as shown in Fig. 3. The full system includes the dynamic of process, which is linear, the elements of controller Kr and K, the disturbances of process, d, and the reference input, r. The objective the controller of feedback is to control the system output,y, that follows the reference input in the disturbances presence.

The significant of the design the controller is performance specification to be stability. We would like the equilibrium point of the system to be stable by absence of the disturbances.

Fig. 3  A state feedback controller

The linear differential equation describes by

x=Ax+Bu

y=Cx,                                                                       (12)

When we take D=0 to be simple and rejected the signal of disturbance d for test. Our significant is to drive the response of output y to a set point,  r .

The control law of time invariant is a function of the set point and the state since the state at time t includes all information essential to estimate the behavior of the future system, it can be described by

u=krr-Kx                                                              (13)

where r is the value of set point.

The control law is relating to block diagram shown in Fig.  . The close loop system got when the feedback is applied to the system (13)is given by

x=Bkrr+(A-BK)x                                             (14)

The problem of control is named the problem of eigenvalue assignment. The value of kr does effect on the solution of steady state but does not effect on the system stability. The output of steady state and equilibrium point for the closed loop system are written as

x=-(A-BK)-1Bkrr,    y=Cx                           (15)

Consequently, kr should be selected such that ,    y=r.

kr= -1(CA-BK-1B)                                                     (16)

The value of  kr is the zero frequency gain inverse of the closed loop system.

By utilizing the gains  kr and  K, we will design the controller of the closed loop system to meet our aim.

By modeling the state feedback control , we can get the values of  A, B, and C. The values of K is 10 and kr is 1.

By simulation the human respiratory system and with state feedback controller, we can get the response of output as shown in Fig. 4 .

Fig. 4 The output with state feedback controller

Now we can use another controller is like PID to compare with response of the output and display what is the best.

The transfer function of PID is

Us=kds2+kps+kise(s)                                     (17)

where  kp, ki, and kd are factors of PID and by tuning the values of these factors, we can get the best response. In this case we chose many values of PID factors and got the best response at values kp, ki,and  kd pi as shown in Fig 5- 7.

Fig. 5 The output with PID (Kp =2.1, ki = 2.1, kd = .0005)

Fig. 6 The output with PID (Kp =10, ki = 1, kd = -.0005)

Fig. 7 The output with PID (Kp =50, ki = 1, kd =.0005)

- CONCLUSION

Modeling of human respiratory system is the main important to study the factors that effects on the lungs, heart ,alveoli , etc. when supply FiO2 inspiratory input between 20%-40% and get the SpO2 saturation output between 90%-95%. To control the SpO2 between ranges 90%-95% with stability response and minimum settling time and zero steady state error, we chose two controllers to compare which one is the best for controller. State feedback controller is more suitable than PID controller. The SpO2 output can get after 50 sec without peak overshoot and zero steady state error when we use state feedback controller but the SpO2 output can get after 550 sec and without peak overshoot and zero steady state error.

.

REFERENCES

- Ireneusz Jabłoński and Janusz Mroczka, “Reduction of a Linear Complex Model for Respiratory System During Airflow Interruption  ,” 32nd Annual International Conference of the IEEE EMBS Buenos Aires, Argentina, August 31 - September 4, 2010.

- Behrang Amini, Akhil Bidani, Joseph B. Zwischenberger, and John W. Clark, “A Model of the Pacemaking Neuron of the Respiratory Central Pattern Generator,” IEEE Transactions on Neural Systems and Rehabilitation Engineering,pp. 120-124 vol. 13, no. 2, June 2005.

- Clara M. Ionescu, Patrick Segers, and Robin De Keyser, “Mechanical Properties of the Respiratory System Derived From Morphologic Insight,” IEEE Transactions on Biomedical Engineering,pp. 949-959, vol. 56, no. 4, April 2009.

- Nicolas W. Chbat, Massimo Giannessi, Antonio Albanese, and Mauro Ursino, “A Comprehensive Cardiopulmonary Simulation Model for the Analysis of Hypercapnic Respiratory Failure,” 31st Annual International Conference of the IEEE EMBS Minneapolis, pp. 5474-5477,Minnesota, USA, September 2-6, 2009.

- Clara Mihaela Ionescu, Ionut Muntean, J. A. Tenreiro-Machado, Robin De Keyser, and Mihail Abrudean, “A Theoretical Study on Modeling the Respiratory Tract With Ladder Networks by Means of Intrinsic Fractal Geometry,” IEEE Transactions of Biomedical Engineering, pp. 246- 253, vol. 57, no. 2, February 2010.

- He zhonghai, “Simulation Study on Human Respiratory System Based on Muscle Pressure Driven,” International Conference on Engineering Computation, pp. 33-35, 2009.

- Mario A. Quiroz-Ju´arez, Omar Jim´enez-Ram´ırez, Rub´en V´azquez-Medina, Elena Ryzhii, Maxim Ryzhiiand Jos´e L. Arag´on, “Cardiac conduction model for generating 12 lead ECG signals with realistic heart rate dynamics,” IEEE Transactions on NonoBioscience, pp. 1-8, vol. 17, no. 4, 2018

- Chi-Hsuan Hsieh,Yu-Fang Chiu, Yi-Hsiang Shen, Ta-Shun Chu, and Yuan-Hao Huang, “A UWB Radar Signal Processing Platform for Real-Time Human Respiratory Feature Extraction Based on Four-Segment Linear Waveform Model,” IEEE Transactions on Biomedical Circuits and Systems, pp. 1-11, vol. 10, no. 1, 2016.

- Micheal Revow, Sandra J. England, Henry O’beirne, and A. Charles Bryan, “A Model of the Maturation of Respiratory Control in the Newborn Infant,” IEEE Transactions on Biomedical Engineering, pp. 414-423, vol. 36, no. 4, 1989.

- Bailiang Chen, Nicolas Weber, Freddy Odille, Claire Large-Dessale, Antoine Delmas, Laurent Bonnemains and Jacques Felblinger, “Design and Validation of a Novel MR-Compatible Sensor for Respiratory Motion Modeling and Correction,” IEEE Transactions on Biomedical Engineering, pp. 1- 11, vol. 64, no. 1, 2017.

- A. Guyton, T. Coleman, and H. Granger, “Circulation: Overall regulation,” Annual Review of Philosophy, vol. 34, no., pp. 13-46, 1972

- Lafta E. J. Alkurawy, “ Neural Network and Control for Arterial Oxygen Saturation in Neonatal Infants,” Journal of      Engineering and Applied Sciences, vol. 14, no. 13, pp. 4532-4541, 2019.

- Lafta Ismaeel Jumaa, “Modelling and Control of Arterial Oxygen Saturation in Neonatal Infants,” Diyala Journal of Engineering Sciences, 2015.

|  | Lafta E. Jumaa Alkurawy1received the B.S., and M.S. degree in Control and systems from Technology University, Baghdad, Iraq, in 1990 and 2003 respectively. He received the Ph.D. degree in Electrical and Computer Engineering from the University of Missouri in Columbia, USA, in 2013. Since 2003, I have been with the University of Diyala, College of engineering, Diyala, Iraq as a lecturer. His current research interests include modeling, control, Numerical analysis and nonlinear |
| --- | --- |
|  | Khalid G. Mohammed is a PhD researcher in the Department of Electrical Power Engineering at University Tenaga Nasional, Malaysia. He has ten years of industrial experience in repairing and maintenance of several types of electrical machines. He has published many papers works in 2007 and 2019 containing many new practical correlations for the design of single and three phase induction motors. He has authored three practical experimentation books in electrical circuits and machines and has been teaching since 2009 in the College of Engineering, Diyala University, Iraq. |
|  | Ammar Issa received his B.Sc from University of Baghdad in Iraq in 2001, MSc from University Tenaga nosional (uniten) in Malaysia at 2013, work at college of engineering University of Diyala, Ira as assistant lecture. His current research interests include power electronic, electrical car, renable energy. |